1) SMS command now supports Camera ID parameter settings:

CameraID: OFF|XXXXXXXXXXXXXX; default is OFF, maximum 14 characters (only the first 14 characters are read if the number exceeds 14, and a space is added at the end if the number is less than 14 characters;

The text content only supports uppercase English, numbers and spaces, and other characters do not respond to Camera ID settings), default is OFF

// For example: turn off the Camera ID function---#OFF#; turn on the Camera ID and set the value---#MYCAMERA000001#

2) The modified software version number is 5MR3POwF334

Turn off camera id: $01*27#1#3#2#5#1#2#0#1#0#1#0#0#0#0#OFF#OFF#OFF#OFF#1#0#1#OFF#OFF#0#0#0#0$

Turn on camera id: $01*27#1#3#2#5#1#2#0#1#0#1#0#0#0#0#OFF#OFF#OFF#OFF#1#0#1#MYCAMERA000001#OFF#0#0#0#0$

Upgrade method

1) Unzip the SD card software upgrade package

2) Copy all files in the software upgrade package to the root directory of the camera's SD card. After copying, reinsert the SD card into the camera

3) Power on the camera and dial to Setup, then click Menu to enter Menu—>Other—>FW Update—>Software—>yes

4) The interface will display "Please wait...", which means that the software upgrade is in progress. At this time, please make sure that the camera is powered normally and do not force the camera to power off

5) After the upgrade is completed, the camera will automatically restart. After that, customers can check whether the software version is correct through Menu—>About—>Long press the right button for 10 seconds. The SD card software upgrade is complete;